Philip McDonald | U37867778 | VLSI Testing Final Project Report

Code Structure

The generator is written in Python and is split into six different files to keep the code organized and modular. The first file is models.py, which defines the basic classes for the program. It includes the Circuit class to hold the netlist data, the Gate class to store gate types, and the Fault class to handle stuck-at faults. The second file is netlist.py which handles actually reading the benchmark files. It reads the file line-by-line, converts the text into Gate objects, and sorts them so the simulation evaluates everything in the correct order

The main circuit logic happens in simulate.py and faults.py. The simulate.py file runs the logic simulation given an input vector. It calculates the binary values for the gates and propagates them to the outputs and it can run a good simulation or a faulty simulation where it forces a specific wire to a stuck-at value. The faults.py file handles the fault collapsing and first isolates fanout branches. Then, it uses equivalence and dominance rules to remove redundant faults and create a smaller list of fault classes.

Finally, the test generation is done in podem.py and main.py. The podem.py file implements the PODEM algorithm using five value logic (0, 1, D, DBar, X). It uses a recursive search to set inputs and propagate the fault to an output. The main.py file acts as the main menu for the user. It takes user input, checks for errors, and calls the functions from the other files to run the simulation, fault collapsing, and test pattern generation.

Compile Instructions

- How to Run the Generator

The generator is written in Python, so it does not need to be compiled like C++ code. It uses only standard libraries so no additional installation is required.

- Requirements

Python 3 or later installed.

- Instructions

- Unzip the project folder.

- Open a terminal or command prompt within the project folder.

- Ensure the benchmark circuit files are located inside the benchmarks folder.

- To run the program, you can use the included Makefile (specific directions are within).

- Follow the on-screen interactive menu to load circuits, perform fault collapsing, or generate test vectors.

Results

- Figure 4.21 (page 117) (t4_21)

- Fault Classes: 10

- Undetectable Faults: 0

- Detailed Output:

1gat:s-a-0 --> Vector: 11000

1gat:s-a-1 --> Vector: 01000

2gat:s-a-1 --> Vector: 10000

3gat:s-a-0 --> Vector: 00100

3gat:s-a-1 --> Vector: 00000

3gat_FO0:s-a-0 --> Vector: 00100

3gat_FO1:s-a-0 --> Vector: 00110

3gat_FO1:s-a-1 --> Vector: 11010

4gat:s-a-1 --> Vector: 00100

5gat:s-a-0 --> Vector: 11001

- Figure 4.3 (page 97) (t4_3)

- Fault Classes: 9

- Undetectable Faults: 0

- Detailed Output:

1gat:s-a-0 --> Vector: 1100

1gat:s-a-1 --> Vector: 0100

1gat_FO0:s-a-0 --> Vector: 1001

1gat_FO0:s-a-1 --> Vector: 0001

1gat_FO1:s-a-1 --> Vector: 0100

2gat:s-a-0 --> Vector: 1100

2gat:s-a-1 --> Vector: 1000

3gat:s-a-0 --> Vector: 1010

4gat:s-a-1 --> Vector: 0000

- Figure 5.10 (page 141) (t5_10)

- Fault Classes: 10

- Undetectable Faults: 0

- Detailed Output:

agat:s-a-0 --> Vector: 11000

agat:s-a-1 --> Vector: 01000

bgat:s-a-1 --> Vector: 10000

cgat:s-a-0 --> Vector: 00100

cgat:s-a-1 --> Vector: 00000

cgat_FO0:s-a-0 --> Vector: 00100

cgat_FO1:s-a-0 --> Vector: 00110

cgat_FO1:s-a-1 --> Vector: 11010

dgat:s-a-1 --> Vector: 00100

egat:s-a-0 --> Vector: 11001

- Figure 5.26a (page 161) (t5_26a)

- Fault Classes: 27

- Undetectable Faults: 0

- Detailed Output:

CI:s-a-0 --> Vector: 101

CI:s-a-1 --> Vector: 100

CI_FO0:s-a-0 --> Vector: 101

CI_FO0:s-a-1 --> Vector: 100

CI_FO1:s-a-0 --> Vector: 001

CI_FO1:s-a-1 --> Vector: 000

L_FO0:s-a-0 --> Vector: 101

L_FO0:s-a-1 --> Vector: 111

L_FO1:s-a-0 --> Vector: 011

L_FO1:s-a-1 --> Vector: 111

L_FO2:s-a-0 --> Vector: 000

L_FO2:s-a-1 --> Vector: 110

N_FO0:s-a-1 --> Vector: 001

N_FO1:s-a-0 --> Vector: 100

N_FO1:s-a-1 --> Vector: 000

T_FO0:s-a-1 --> Vector: 101

T_FO1:s-a-1 --> Vector: 101

T_FO2:s-a-1 --> Vector: 101

X:s-a-0 --> Vector: 111

X:s-a-1 --> Vector: 011

X_FO0:s-a-0 --> Vector: 111

X_FO0:s-a-1 --> Vector: 011

X_FO1:s-a-1 --> Vector: 001

Y:s-a-0 --> Vector: 111

Y:s-a-1 --> Vector: 101

Y_FO0:s-a-1 --> Vector: 101

Y_FO1:s-a-1 --> Vector: 001

- Figure 6.24 (page 200) (t6_24_v1)

- Fault Classes: 37

- Undetectable Faults: 6

- Detailed Output:

17gat_FO0:s-a-0 --> Vector: 000010

17gat_FO0:s-a-1 --> Vector: 111111

17gat_FO1:s-a-0 --> Vector: 000100

17gat_FO1:s-a-1 --> Vector: 111111

17gat_FO2:s-a-0 --> Vector: 000001

17gat_FO2:s-a-1 --> Vector: 111111

1gat:s-a-0 --> Vector: 100000

1gat:s-a-1 --> Vector: 000000

1gat_FO0:s-a-1 --> Vector: 000000

1gat_FO1:s-a-0 --> Vector: 111111

1gat_FO1:s-a-1 --> Vector: 011111

2gat:s-a-0 --> Vector: 010000

2gat:s-a-1 --> Vector: 000000

2gat_FO0:s-a-1 --> Vector: 000000

2gat_FO1:s-a-1 --> Vector: 101111

3gat:s-a-0 --> Vector: 111111

3gat:s-a-1 --> Vector: 110111

3gat_FO0:s-a-1 --> Vector: 110111

3gat_FO1:s-a-1 --> Vector: 000000

4gat:s-a-0 --> Vector: 000100

4gat:s-a-1 --> Vector: 000000

4gat_FO0:s-a-1 --> Vector: 000000

4gat_FO1:s-a-0 --> Vector: 111111

5gat:s-a-0 --> Vector: 000010

5gat:s-a-1 --> Vector: 000000

5gat_FO0:s-a-1 --> Vector: 000000

5gat_FO1:s-a-0 --> Vector: 111111

6gat:s-a-0 --> Vector: 111111

6gat:s-a-1 --> Vector: 111110

6gat_FO0:s-a-0 --> Vector: 111111

6gat_FO2:s-a-1 --> Vector: 000000